Formative Assessment Report on
“Dust Detection and Maintenance Optimization for

Solar PV  Panels using CNN-RF and I-V Curve Data”

Submitted towards the fulfilment of the requirement of
Fundamentals of Digital Image Processing
Bachelor of Technology(B-Tech)

Academic year 2025-2026
(Third Year)

Submitted by:
Jai Chandegave     123B1F016

Under the Guidance
Dr.Harsha Bhute

Intelligent Automated Dust Detection and Maintenance Optimization for Solar PV Panels using CNN-RF and I-V Curve Data

Aim:

To design an intelligent system using CNN-RF and I–V curve data for automatic dust detection and optimized cleaning of solar panels, improving efficiency and reducing maintenance costs.

Problem Statement:

Dust accumulation on PV panels may decrease their energy production by up to 30% within a single month. Efficiency and economic viability are impacted by this. Hand cleaning and checking these things takes a lot of work, costs a lot, and uses up lots of water, especially when they're big solar farms in places that don't get much rain. Current detection methods frequently fail to precisely gauge dust levels, require specific weather conditions for accuracy, and inadequately integrate cleaning techniques.

Motivation:

• Solar power must become more dependable and effective in order to assist in achieving  worldwide environmental objectives.

• Traditional PV maintenance requires a lot of water, which raises costs and causes environmental problems.

• No current method provides both automatic, accurate dust identification and combined, efficient cleaning that happens instantly.

Objectives:

• Developing a hybrid CNN-RF system for accurate operational state classification and dust severity detection.

• To turn I-V curve info into colorful pictures that show important features well, even in nature's way.

• To develop an economical and efficient cleaning system linked to measured dust intensity.

Background:

Solar panels help generate clean power globally, yet dirt buildup reduces their efficiency significantly. Manual upkeep techniques lack efficacy and longevity. Computer vision, machine learning, and Internet of Things have led to automatic monitoring and maintenance of photovoltaic panels. This research introduces an intelligent system capable of identifying dust, categorizing its level of hazard, and enhancing cleaning efficiency. This strategy aids in achieving SDGs 7 (affordable energy), 12 (responsible consumption), and 13 (climate action).

Related Work :

| Study | Method | Description |
| --- | --- | --- |
| Abukhait (2024) | GLCM + LBP + SVM | Used visible light imaging and combined gray level co-occurrence matrix (GLCM) textural features with local binary patterns (LBP). Classification was done using a support vector machine (SVM) and achieved up to 94.3% accuracy for moderate to heavy dust. Integration with UAV or drone-based monitoring systems was proposed. |
| Onim et al. (2022)) | CNN (SolNet) | Developed and applied a custom convolutional neural network, SolNet, for image-based dust detection. This approach achieved 98.2% accuracy in distinguishing dusty panels from clean ones. The model was trained on a dataset of 2,231 images and validated for situations with light and typical accumulation. |
| Bashir et al. (2025) | CNN-RF with RGB Mosaic Images | Gave a new hybrid system that converts I-V curve data into RGB mosaic images. This process is handled by CNN and classified with a random forest (RF). Achieved 100% accuracy for operational states and 98% for dust severity. Also introduced automated, cost-effective cleaning recommendations that use water and loss cost factors. |
| Unluturk et al. (2019) | GLCM + ANN | Used artificial lighting and image processing to estimate dust accumulation levels (three levels) by extracting GLCM texture features and using artificial neural networks (ANN). Achieved 96.86% accuracy in lab scenarios |

Methodology:

A comprehensive approach that involves multiple steps and integrates all aspects of the system to detect dust accumulation on solar panels, assess its impact, and initiate an economical cleaning process. The method combines I-V curve analysis, mixed machine intelligence, and self-guided operations.

- Data Acquisition and transformation details

• The I-V terminal voltage, current, and power measurements will use precise analyzers or built-in I-V tracers within inverters. Data will be collected under various PV panel conditions such as normal, shaded, shaded, degraded, contaminated, and dusted states.

• RGB Mosaic: Electrical measures were standardized to have a small range, then divided among each color's channels, producing distinct RGB triplets. These are shown and described in a grid format (two-dimensional image), illustrating the panel's condition and allowing visual input into related electrical data.

- Extraction of Features & Classification

• Primary CNN-RF Model: The RGB mosaic possesses intricate spatial characteristics captured by a Convolutional Neural Network (CNN), resulting in high-dimensional data. The derived features were used with a Random Forest (RF) classifier to determine whether the solar panel was in normal operation, due to shading, experiencing degradation, having a short-circuit fault, or being covered by dust.

• Secondary CNN-RF Model: The secondary model runs only if dust is identified in the primary model. The secondary model classifies dust accumulation as mild, moderate, and severe.

- Cleaning Optimization Framework

• A Cost-Benefit Decision Model uses a dynamic optimization algorithm to balance the economic impact of reduced power output against the expense of cleaning, focusing mainly on water usage and labor costs.

• The framework suggests an economic cost for cleaning that varies according to how dirty something is classified. The amount of water needed for each module varies (two liters for low dust, five liters for moderate dust, and ten liters for high dust). Partial cleaning may also be possible, aiming to minimize resource usage that might otherwise go unused.

- Autonomous Deployment

• The detection and decision-making system is installed on an unmanned aerial vehicle, or robot platform.

• The algorithm employs an optimized A method to chart a route for the drone inspection that fulfills all required spatial criteria, ensuring coverage of the solar farm efficiently and swiftly, and reducing both power consumption and inspection times.

Conclusion :

This study was selected to respond to the urgent need for a resource-efficient and automated PV maintenance solution due to the ever-increasing solar uptake and environmental limitations. Utilizing state-of-the-art machine learning methods and IoT methodology in the research, the study goal was reached effectively:

The near-perfect accuracy of panel state and evidence of dust severity detection.

The reduction of water and operational costs by optimizing cleaning schedules. A scalable and robust methodology that can be applied in actual commercial solar farms.

Some of the key take-aways from this research include the strong performance of RGB mosaic image-based deep learning classification and the importance of making cleaning decisions based on detection outputs. The gaps in previous research (fine-grained dust severity detection, cleaning optimization) were directly addressed in the proposed system.

The next step will be to deploy the system in the real-world conditions, advance the algorithms, and incorporate measuring with respect to other environmental factors for perhaps even more intelligent maintenance strategies.